USB Info Tool -Documentation

Balc Dragoș, Birescu Ionuț, Dobra Darius, Georgescu Rareș

I. About USB Flash Memory

1. The Physical Target: NAND Flash Memory

Our application interacts with USB Mass Storage Devices. Unlike traditional Hard Disk Drives (HDDs) which use magnetic spinning platters, the hardware we are targeting utilizes NAND Flash Memory.

- Storage Cells: The data accessed by our ls (list files) command is stored in microscopic cells on the USB’s silicon chip.

- Write Cycles: The speed command in our code performs direct Write Operations. It is important to note that Flash memory has a limited lifespan (write endurance). Our speed test writes a temporary file to test the memory controller's efficiency, then deletes it to preserve space.

2. Data Transfer Architecture

When the user executes commands in our main.cpp, the data flows through a specific hardware pipeline:

- Application Layer: The user types of list or speed.

- OS Kernel: Windows converts our C++ requests into SCSI / USB Mass Storage Commands.

- Host Controller: The computer's hardware (USB 3.0/3.1 port) sends electrical signals.

- Device Controller: The small processor inside the USB stick receives the signal.

  - For ls: It reads the FAT Table (Sector 0-32) to find file names.

  - For speed: It manages high-voltage operations to program the flash cells.

3. Hardware Performance Testing (command == "speed")

The code includes a specific module USBTester::TestSpeed. This feature is designed to benchmark the physical interface speed.

- USB 2.0 vs 3.0: By running this command, we can physically identify the hardware generation:

  - If Write is ~5-20 MB/s $\rightarrow$ The hardware is likely USB 2.0.

  - If Write is >50 MB/s $\rightarrow$ The hardware is likely USB 3.0/3.1.

- Bottlenecks: The speed test reveals hardware bottlenecks. If the "Read" speed is high but "Write" is low, it indicates a limitation in the USB stick's NAND Controller (it cannot clear blocks fast enough), rather than the USB port itself.

4. Logical Addressing (LBA)

USB hardware is organized into 512-byte Sectors.

- Our application abstracts this. We do not read raw electricity; we rely on the Logical Block Addressing (LBA) provided by the USB controller.

- When usbManager.detectUSBDevices() runs, it queries the hardware to see how many usable Logical Blocks exist, which we then calculate into Gigabytes (GB) for the user display.

II. About Our Application

The USB Information Tool is a Windows-based application developed in C/C++ as a student team project. Its main purpose is to explore how software applications can interact with USB mass storage devices through the operating system. The tool detects connected USB flash drives and displays essential information such as the device label, total capacity, free space, and basic file system details.

The application is composed of two components:

- a console application, used mainly for testing and validating core functionality,

- a graphical user interface (GUI), which offers a more intuitive and interactive way to manage USB devices.

Together, these components showcase practical USB device handling, file system operations, and performance measurement in a real Windows environment.

Graphical Application Overview (Win32 GUI)

In addition to the console version, the project includes a native Windows GUI application implemented using the Win32 API. The graphical interface provides a user-friendly way to interact with USB storage devices. It automatically detects removable drives, allows users to browse folders and files, and supports basic file management operations such as creating folders, creating files, renaming entries, and deleting them.

One of the key features of the GUI is the USB speed benchmark, which measures read and write performance directly on the selected device. This feature helps visualize how hardware characteristics affect real-world data transfer speeds.

Why Win32 API?

The Win32 API was chosen because it:

- runs without external GUI frameworks, making it lightweight and easy to deploy in a lab environment,

- provides direct access to Windows system messages such as WM_DEVICECHANGE, allowing real-time detection of USB insertion and removal,

- integrates smoothly with the existing C++ backend modules (USBManager, USBTester, Logger).

GUI Code Structure and Responsibilities (main_gui.cpp)

The GUI follows the classic event-driven programming model used by Win32 applications.

- WinMain(...)
Initializes the logging system, registers the window class, creates the main application window, and starts the message loop.

- WndProc(...)
Acts as the central event handler. It processes button clicks, double-click actions in the list view, device change notifications, and interface updates.

- Helper functions
Handle common tasks such as formatting file sizes, managing ListView columns and rows, filtering content, and sorting entries.

Application Startup Flow

When the GUI application starts:

- Logger::Init("usbtool.log") creates or opens the log file.

- The window class (USBManMax) is registered with Windows.

- The main window is created, centered on the screen, and titled “USB Manager Ultimate (+File)”.

- The message loop begins and runs until the application exits.

Logging the startup process is essential for debugging and for correlating user actions with system-level events.

User Interface Components

1) Main List View (Device / File Browser)

The core interface element is a ListView control in report mode with three columns:

- Name – drive label or file/folder name

- Type – “Drive”, “Folder”, or “File”

- Size / Usage – file size or drive free-space information with a visual usage bar

This unified layout ensures a consistent experience whether the user is viewing drives or browsing directories.

2) Command Buttons

The top command bar provides the following actions:

- Back – navigates up one level or returns to drive selection

- + Folder – creates a new folder using an automatic naming strategy

- + File – creates a new file with a user-defined name

- Rename – renames the selected file or folder

- Delete – deletes the selected file or folder

- Speed Test – runs a performance benchmark on the selected USB drive

Buttons are dynamically enabled or disabled based on context:

- In drive selection mode, file operations are disabled.

- In directory browsing mode, all relevant actions become available.

This prevents invalid actions and improves overall usability.

3) Search Box (Live Filtering)

The search field filters displayed entries in real time:

- In drive mode, it filters by drive letter and label.

- In directory mode, it filters by file or folder name.

Filtering is triggered on each text change event, providing immediate feedback as the user types.

Navigation Logic (Drive Selection → Directory Browsing)

The GUI logic is centered around a single state variable:

std::string currentPath = "";

- When currentPath is empty, the application displays removable drives detected via usb.detectUSBDevices().

- When currentPath is set (e.g., "E:\\"), the application displays directory contents using usb.getFiles(currentPath).

Double-Click Behavior

- Drive mode: double-clicking a drive enters directory browsing mode.

- Browsing mode:

  - double-clicking a folder navigates into it,

  - double-clicking a file opens it using ShellExecute, relying on the system’s default application.

This behavior closely mirrors Windows Explorer and feels intuitive to users.

Sorting and Display Formatting

Column Sorting

Clicking a column header sorts the displayed items:

- repeated clicks toggle ascending and descending order,

- sorting logic adapts based on whether the user is viewing drives or directory contents.

Size and Usage Formatting

Helper functions improve readability:

- FormatSize(bytes) converts raw byte values into human-readable units,

- CreateProgressBar(percent) displays a simple text-based usage bar, offering an at-a-glance view of drive capacity.

Input Dialog and the Anti-Freeze Design

Instead of using blocking console input, the GUI uses a custom input dialog:

- GetUserInput(parent, title, outBuf)

To prevent the application from freezing:

- the parent window is temporarily disabled,

- a local message loop runs while the dialog is active,

- the parent window is re-enabled afterward.

This approach avoids UI lockups and follows best practices for Win32 application design.

File Operations and Backend Mapping

The GUI does not implement file system logic directly. All operations are delegated to the USBManager module:

- folder creation → usb.createAutoFolder(...)

- file creation → usb.createFile(...)

- renaming → usb.renameEntry(...)

- deletion → usb.deleteFile(...)

This separation keeps the GUI lightweight and allows both the console and GUI applications to share the same backend logic.

Speed Test Feature and Multithreading

How the Speed Test Works

When the user clicks Speed Test:

- The drive letter is extracted from currentPath.

- The button is disabled to prevent repeated clicks.

- The action is logged.

- A worker thread runs USBTester::TestSpeed(...).

Why Multithreading Is Necessary

Disk I/O operations can take several seconds. Running them on the main UI thread would freeze the interface. Instead:

- the benchmark runs in a detached thread,

- the UI is safely updated via PostMessage,

- the main thread re-enables the button once the test finishes.

This demonstrates good software engineering practices regarding responsiveness and thread-safe UI updates.

Automatic USB Refresh

The GUI listens for WM_DEVICECHANGE events. When a USB device is inserted or removed, the view refreshes automatically, ensuring that the displayed drive list is always up to date without manual rescanning.

III. Testing our Application

USBTester Module Overview

The USBTester module measures real-world USB performance by:

- creating a temporary test file on the selected USB device,

- performing controlled write and read operations,

- timing the operations using std::chrono,

- calculating throughput in MB/s.

To ensure accurate results, the module uses:

- large data chunks to reduce overhead,

- aligned memory buffers,

- write-through and no-buffering modes to minimize OS caching effects.

After the test completes, all resources are cleaned up and results are logged. Any errors, such as device removal during testing, are also recorded.

Final Remarks

Although written in C++, the application does not interact directly with hardware signals. Instead, it relies on Windows APIs that translate software requests into operations handled by the operating system’s USB and storage subsystems. Through this approach, the project provides a realistic and practical demonstration of how software, the operating system, and hardware work together.

This makes the USB Information Tool a valuable learning project that combines low-level system interaction, user interface design, and real hardware performance analysis.